- MATERIAL SAFETY DATA SHEET

SECTION 1 – PRODUCT IDENTIFICATION AND USE

Product Identifier: Jokisch Econo Paste 897      ProductID(PIN):

- Product Use: Tapping and Cutting Paste

Manufacturer’s Name:                      Supplier’s Name:

Jokisch GmbH                              Intercon Enterprises Inc.

11-7550 River Road

Delta, BC V4G 1C8

Germany                                   Canada

Tel:                                      Tel: 604 946 6066

Fax:                                      Fax: 604 946 5340

# SECTION 2 – INGREDIENTS

Ingredients                           %     CAS Number     LD50/LC50 of Ingredients

10-25%  64742-01-4     28600 mg/kg oral (rats)

10-25%  73138-44-0

10-25%  85535-85-9

20-40%  8001-31-8

20-40%  68425-15-0

# SECTION 3 – PHYSICAL DATA

Physical State: paste

Odour & Appearance: neutral odour, light brown paste

Odour Threshold (ppm): n.av.

Vapour Pressure (mm Hg): n.av.

Vapour Density (AIR=1): n.av.

Evaporation Rate: n.av.

Boiling Point: n.av.

Freezing Point: n.app.

pH: n.app.

Specific Gravity (g/ccm): 1.00 Method DIN 51 757

Coeff. Water/Oil Dist.: n.av.

# SECTION 4 – FIRE AND EXPLOSION DATA

Flammability: No

Means of Extinction: Foam, Halon, Dry Chemical

Flash Point: 250oC Method DIN ISO 2592

Upper Flammable Limit: n.av.

Lower Flammable Limit: n.av.

Auto Ignition Temperature: 320oC Method DIN 51794

Hazardous Combustion Products: HCl (hydrogen chloride) & S02 (sulphur dioxide)

Explosion Data: n.av.

Sensitivity to Static Discharge: n.app.

## Product Identifier: Jokisch ECONO 897

# SECTION 5 – REACTIVITY DATA

Chemical Stability: Yes

Incompatibility with Other Substances: No

Reactivity and Under What Conditions: extreme heat approx. 180oC – 200oC

Hazardous Decomposition Products: HCl (hydrogen chloride) & S02 (sulphur dioxide)

# SECTION 6 – TOXICOLOGICAL PROPERTIES

Route of Entry: eye contact

Effects of Acute Exposure: mild irritant when rubbed in eyes

Effects of Chronic Exposure to Product: n.av.

Exposure Limits: n.av.

Irritancy of Product: mild irritant when rubbed in eyes

Sensitization to Product: n.av.

Carcinogenicity: n.av.

Teratogenicity: n.av.

Reproductive Toxicity: n.av.

Mutagenicity: n.av.

Synergistic Properties: n.av.

# Section 7 – PREVENTIVE MEASURES

Personal Protective Equipment:

Gloves (specify): avoid contact with skin wear gloves

Respirator (specify): n.app.

Eye (specify): avoid contact with eyes wear safety glasses

Footwear (specify): n.app.

Clothing (specify): n.app.

Other (specify):

Engineering Controls: local ventilation. Exhaust only necessary if measurements above 5 mg/m3

Leak & Spill Procedure: pick up with binding agent and treat as waste oil

Waste Disposal: dispose of according to Federal, Provincial & local regulations

Handling Procedure: apply manually. Do not allow oil to enter ground water

Storage Requirements: keep container closed and avoid direct heat exposure

Special Shipping Information: none

# Section 8 - FIRST AID MEASURES

Specific Measures:

Inhalation: if worker is overcome via misuse, bring person to fresh air, call physician

Ingestion: call physician and refer to MSDS

Eyes: rinse with plenty of water, call physician

Skin: general hygiene, skin protective ointment, wash with soap & water

Information: safety data from manufacturers of components and Jokisch GmbH

# SECTION 9 – PREPARATION DATE OF MSDS

Prepared By: Intercon Enterprises Inc.

Phone Number: 604 946 6066

Date: January 2003