- MATERIAL SAFETY DATA SHEET

SECTION 1 – PRODUCT IDENTIFICATION AND USE

Product Identifier: Jokisch SR-X1 Concentrate    ProductID(PIN): 1824

- Product Use: Water-soluble system cleaner

Manufacturer’s Name:                      Supplier’s Name:

Jokisch GmbH                              Intercon Enterprises Inc.

11-7550 River Road

Delta, BC V4G 1C8

Germany                                   Canada

Tel:                                      Tel: 604 946 6066

Fax:                                      Fax: 604 946 5340

# SECTION 2 – HAZARDOUS INGREDIENTS

Hazardous Ingredients                 %     CAS Number     LD50/LC50 of Ingredients

1,3,5 Triazin 2H 4H 6H –Triethanol   8-12%  4719-04-4

Sodium Hydroxide                     4.5%   1310-73-2

Pyridin-2-thiol-1-oxid               <1%    3811-73-2

# SECTION 3 – PHYSICAL DATA

Physical State: liquid

Odour & Appearance: light yellow, ammonia like odour

Odour Threshold (ppm): n.av.

Vapour Pressure (mm Hg): 20 mbar @ 20oC

Vapour Density (AIR=1): n.av.

Evaporation Rate: n.av.

Boiling Point: 100oC

Freezing Point: 0oC

pH: 12 @ 1% solution with water

Specific Gravity (g/ccm): 1.08 Method DIN 51 757

Coeff. Water/Oil Dist.: n.av.

# SECTION 4 – FIRE AND EXPLOSION DATA

Flammability: No

Means of Extinction: Foam, Halon, Dry Chemical, Water

Flash Point: >100oC Method ISO 2715

Upper Flammable Limit: n.av.

Lower Flammable Limit: n.av.

Auto Ignition Temperature: n.av.

Hazardous Combustion Products: Formaldehyde (small quantities)

Explosion Data: n.av.

Sensitivity to Static Discharge: n.app.

## Product Identifier: Jokisch SR-X1 Concentrate

# SECTION 5 – REACTIVITY DATA

Chemical Stability: Yes

Incompatibility with Other Substances: Yes with acids

Reactivity and Under What Conditions: n.av.

Hazardous Decomposition Products: Formaldehyde

# SECTION 6 – TOXICOLOGICAL PROPERTIES

Route of Entry: eye contact, skin contact, skin adsorption, inhalation, ingestion

Effects of Acute Exposure: known irritant to skin eyes and inhalation

Effects of Chronic Exposure to Product: n.av.

Exposure Limits: n.av.

Irritancy of Product: irritant skin eyes and inhalation

Sensitization to Product: n.av.

Carcinogenicity: n.av.

Teratogenicity: n.av.

Reproductive Toxicity: n.av.

Mutagenicity: n.av.

Synergistic Properties: n.av.

# Section 7 – PREVENTIVE MEASURES

Personal Protective Equipment:

Gloves (specify): rubber gloves are necessary

Respirator (specify): disposable, half face piece (fume, vapour type) air filter

Eye (specify): goggles are necessary

Footwear (specify): n.app.

Clothing (specify): change wet clothes

Other (specify):

Engineering Controls: local ventilation. DO NOT INHALE VAPOURS

Leak & Spill Procedure: pick up with binding agent and treat as waste oil

Waste Disposal: dispose of according to Federal, Provincial & local regulations

Handling Procedure: apply manually or with pump system. Do not allow oil to enter ground water

Storage Requirements: keep container closed and avoid freezing

Special Shipping Information: may desire heated shipping during winter months

# Section 8 - FIRST AID MEASURES

Specific Measures:

Inhalation: on suspicion of inhalation to lungs, immediately call physician

Ingestion: call physician, do not induce vomiting, and refer to MSDS

Eyes: rinse with plenty of water, call physician

Skin: general hygiene, skin protective ointment, wash with soap & water

Information: safety data from manufacturers of components and Jokisch GmbH

# SECTION 9 – PREPARATION DATE OF MSDS

Prepared By: Intercon Enterprises Inc.

Phone Number: 604 946 6066

Date: January 2003